# ANALYSIS OF PERFORMANCE CHARACTERISTICS OF THE PROCESSOR FOR VARIED CACHE SIZES USING SIMPLESCALAR AND SPEC 2000 BENCHMARK SUITE

Nitin Thota(2354187)

1. ABSTRACT: The evaluation and analysis of the cache's performance involve utilizing the SimpleScalar "sim-outorder" model in conjunction with the SPEC-2000 benchmark suite. This analysis entails the systematic variation of parameters such as cache size, instruction and data size, unified cache size, and block size.

Index Terms: SimpleScalar, SPEC 2000 benchmark suite

2. INTRODUCTION:

The SimpleScalar toolset serves as a system software infrastructure designed for program performance analysis. It enables the development of various applications for tasks such as microarchitectural modeling and hardware-software co-verification. Notably, simulation with SimpleScalar yields valuable insights, with performance evaluation standing out as one of its key outcomes.

3. OVERVIEW:

SimpleScalar constitutes a set of processor simulators and associated utilities, with PISA being the designated simulation architecture. Among these simulators, "sim-outorder" specifically emulates a superscalar processor with out-of-order execution at the instruction level. It supports speculative execution, and its memory system operates on a two-level hierarchy.

This performance simulator meticulously tracks the latency of each pipeline action, working diligently to monitor both latency and resource usage, which results in a noticeable slowdown. Despite the fact that the host processor executes approximately 2 billion instructions per second, "sim-outorder" processes only about 50 million instructions per second, rendering it approximately 4000 times slower than a real processor. During simulation runs, the tool may remain silent for extended periods.

Additionally, the SPEC CPU2000 benchmark suite comprises 26 non-trivial, compute-intensive programs. The primary purpose of these programs is to evaluate a computer's performance in terms of CPU, memory system, and compiler efficiency.

The benchmarks in this suite display a variety of runtime behaviors because they were chosen to simulate real-world applications. Ten benchmarks are used in this project: gcc, bzip, mesa, eon, wupwise, galgel, crafty, mcf, and swim.

4. METHODOLOGY:

To find the cache performance (i.e., how the cache parameters affect performance), I modified configurations and simulated the data using SimpleScalar sim-outorder with 10 different programs. The performance evaluation is based on 100 million instructions.

Default Cache Configurations:

- Data L1 cache: 16KB with 128 sets, 32B block size, and 4-way associative.

- Instruction L1 cache: 16KB with 512 sets, 32B block size, and 1-way associative.

- Unified L2 cache: 256KB with 1024 sets, 64B block size, and 4-way associative.

Modified Cache Configurations:

- Instruction L1 Cache:

  - Sizes: 8KB, 16KB, 32KB, and 64KB (block size and associativity constant at 32B and 1, respectively).

- Data Cache:

  - Sizes: 2KB, 4KB, 32KB, and 128KB (block size and associativity constant at 32B and 4, respectively).

- Data Cache Associativity:

  - Levels: 1-way, 2-way, 4-way, and 8-way (cache size and block size constant at 32KB and 32B, respectively).

- L2 Cache:

  - Sizes: 64KB, 128KB, 512KB, and 1024KB (block size and associativity constant at 64B and 4, respectively).

- Block Sizes (L1 and L2):

  - Sizes: 32B, 64B, 128B, and 256B (default cache size and associativity constant).

5. SIMULATION RESULTS:

5.1

5.2

5.3data

5.4

1. 1 way…

2.

3.

Dc l2 size

5.block size

Ld and store

hierarchy. This performance simulator meticulously tracks the latency of each pipeline action, working diligently to monitor both latency and resource usage, which results in a noticeable slowdown. Despite the fact that the host processor executes approximately 2 billion instructions per second, "sim-outorder" processes only about 500,000 instructions per second, rendering it approximately 4000 times slower than a real processor. During simulation runs, the tool may remain silent for extended periods. Additionally, the SPEC CPU2000 benchmark suite comprises 26 non-trivial, compute intensive programs. The primary purpose of these programs is to evaluate a computer's performance in terms of CPU, memory system, and compiler efficiency. The benchmarks in this suite display a variety of runtime behaviors because they were chosen to simulate real-world applications. 10 benchmarks are used in this project, and they are gcc, bzip, mesa, eon, wupwise, galgel, crafty, mcf, swim. 4. METHODOLOGY: To find the cache performance i.e., how the cache parameters affect the performance I changed few configurations and simulated the data using SimpleScalar sim-outorder with 10 different programs. The default configurations of the L1 and L2 cache are: ➢ Data L1 cache: 16KB with 128 sets, 32B block size and 4-way associative. ➢ Instruction L1 cache: 16KB with 512 sets, 32 block size and 1-way associative. ➢ Unified L2 cache: 256KB with 1024 sets, 64 block size and 4-way associative. The configurations are changed as follows: ✓ Instruction L1 cache is taken as 8KB, 16KB, 32KB and 64KB keeping the block size and associativity constant as 32B and 1 respectively. ✓ Data cache size is taken as 2KB, 4KB 32KB, 128KB keeping the block size and associativity constant as 32B and 4 respectively. ✓ Data cache associativity is